Futurecom Systems Group, ULC

Vehicular Repeater (VR)

User’s Guide

Document: 8A093X01

Revision: R1.2

Date: November 2021

# TABLE OF CONTENTS

Customizing User Guide Template	4

VR Quick Reference Card (Template)	5

VR Quick Reference Card (Sample)	6

Declaration of Conformity	7

Important Safety Information	7

Notice To Users (FCC/ISED)	7

Déclaration de Conformité	8

Informations de Sécurité Importantes	8

Avis Aux Utilisateurs (FCC/ISED)	8

Proprietary Statement	9

Getting Started	10

How To Use This Guide	10

Notations Used In This Guide	10

System Topology	11

VR Introduction	12

VR Modes	14

OFF Mode (Mobile Mode)	14

LOCAL Mode	14

SYSTEM Mode	14

DVRS Equipment	16

DVRS Configurations	17

VR Operation	18

How To Power Up VR	18

How To Activate VR	18

How To Deactivate VR	20

How To Power Down VR	21

How To Navigate Control Head Menu	22

DVRS Suitcase Deployment	35

Deployment Instructions	35

DVRS Feature Overview	36

DVRS FEATURE DESCRIPTIONS	36

References	39

Appendix A: All VR Status Tones- P25 PSU	40

Appendix B: All VR Status Tones- Conventional Analog PSU	41

Glossary	43

# Customizing User Guide Template

This document is intended to be used as a starting point for customers to create their own User Guide. It may be used as is, with minor modifications, or may be customized to reflect the exact configuration of equipment.

STEP 1: Remove any paragraphs or figures that do not apply to specific customer configuration, if desired.

STEP 2: Update any configurable parameter information identified with the tools icon to reflect specific customer configuration, if desired.

Related parameter is configurable- update with value specific to actual equipment configuration.

STEP 3: Replace control head pictures with actual screen shots or customized channel names, if desired. Use the template provided here to customize control head display. There are 4 elements: VR Status Icon, Channel Text Box, Menu Button Text Box and the Main Display.

TEXT       TEXT       TEXT      TEXT

VR STATUS ICON

CHANNEL TEXT BOX

MENU BUTTON TEXT BOX

MAIN DISPLAY

CUSTOMIZE AS NEEDED

VR MODE VR CHANNEL NAME

MSU CHANNEL NAME

STEP 4: Create a VR Quick Reference Card using template provided with details that match specific customer configuration. Or, use the sample provided- just delete the word “SAMPLE”.

STEP 6: Delete this instruction page along with the VR Quick Reference Card template (if not used) and example (if not used).

STEP 7: Update the Table of Contents to reflect the customized User Guide content.

# VR Quick Reference Card (Template)

INSERT COMMONLY HEARD VR TONES

INSERT CUSTOMIZED VR MODE DIAGRAM

INSERT TRANSMIT AND RECEIVE VR ICONS

INSERT VR STATUS DISPLAY MESSAGES

INSERT PRIMARY AND SECONDARY VR ICON SYMBOLS

ADD PERMANENT PRIMARY IF APPLICABLE

INSERT KEY OPERATIONAL NOTES FOR USER

NOTE:

INSERT PICTURE OF CONTROL HEAD IDENTIFYING LOCATION OF VR INFORMATION

INSERT KEY HOW TO INSTRUCTIONS

HOW TO…

- Step 1

- Step 2

- Step 3

# VR Quick Reference Card (Sample)

NOTE: VR and PSU must be on same VR Channel to enable communication through DVRS.

NOTE: For vehicle-mount, VR must be OFF or a VR Disabled MSU channel selected when vehicle is moving.

HOW TO CHANGE MODE VIA CONTROL HEAD

- Press DVRS button

- Press MODE button until VR SYS, VR LOC or OFF displayed

- Press SEL button or wait 6 seconds

HOW TO CHANGE CHANNEL VIA CONTROL HEAD

- Press DVRS button

- Press Channel Change button

- Use navigation buttons or rotary knob on control head to highlight channel

- Press SEL button or wait 6 seconds

REMOTE ACTIVATION VIA LOCAL DIGITAL PSU

- send a Call Alert Page using the ID of the specific VR

- OR initiate an Emergency Alarm

VR OFF             VR LOCAL             VR SYSTEM

| TONE NAME | TONE HEARD ON PSU |
| --- | --- |
| VR Activation Tones | Three loud tones |
| PSU Go-Ahead Tones: Local Access | Three medium pitched quick chirps |
| PSU Go-Ahead Tones: System Access | Three high pitched quick chirps |
| PSU Fallback (Talkaround) | One long medium pitched tone followed by four high pitched chirps |
| PSU Out of Range | One long medium pitched tone |

| DISPLAY MESSAGE | ACTION |
| --- | --- |
| VR SERVICE | Wait until current activity has completed. The status message will clear once VR is operational. |
| VR ERROR | Radio technician support required to resolve error. |
| VR IN RESET | Wait until restart is complete. The status message will clear once VR is operational. |
| VR DISABLED | Select a VR Enabled Channel and the status message will clear. |
| VR WARNING | Radio technician support required to resolve warning. |

| DISPLAY ICON | VR STATUS |
| --- | --- |
|  | VR Receiving |
|  | VR Transmitting |
|  | VR Receiving and Transmitting |

| DISPLAY ICON | VR STATUS |
| --- | --- |
| Steady | Primary (Active) |
| Flashing | Secondary (Standby) |
| Steady | Active Permanent Primary |
| Flashing | Standby Permanent Primary |

# Declaration of Conformity

This equipment has been tested and found to comply with the limits for a Class A digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment. This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instruction manual, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation.

Changes or modifications not expressly approved by Futurecom Systems Group, ULC. could void the User’s authority to operate the equipment.

# Important Safety Information

The DVRS Repeater is intended for use in occupational / controlled conditions, where users have full knowledge of the operator exposure and can exercise control over the operator exposure to meet FCC/ISED limits. This radio is NOT authorized for general population, consumer, or any other use.

# Notice To Users (FCC/ISED)

To satisfy FCC/ISED RF exposure requirements for mobile transmitting devices, refer to the RF Safety Booklet[1] for TX – RX duty cycle and a separation distance between the antenna of this device and persons during operation. To ensure compliance, operations at closer than this distance is not allowed.

Futurecom requires the P25 DVRS operator to ensure FCC/ISED Requirements for Radio Frequency Exposure are met. The minimum distance between all possible personnel and the body of the DVRS equipped vehicle is specified in the RF Safety Booklet[2]. Failure to observe the Maximum Permissible Exposure (MPE) distance exclusion area around the antenna may expose persons within this area to RF energy above the FCC exposure limit for bystanders (general population).

It is the responsibility of the repeater operator to ensure that MPR limits are always observed during repeater transmissions. The repeater operator must always ensure that no person comes within MPE distance from the antenna.

# Déclaration de Conformité

Cet équipement a été testé et déclaré conforme aux limites pour appareils numériques de classe A, selon la partie 15 des règlements de la FCC. Ces limites sont destinées à assurer une protection raisonnable contre les interférences nuisibles dans une installation commerciale. L’équipement génère, utilise et peut émettre de l’énergie de fréquence radio et peut causer des interférences nuisibles aux communications radio s’il n’est pas installé ou utilisé conformément au mode d’emploi. Toutefois, rien ne garantit l’absence d’interférences dans une installation particulière.

Les changements et les modifications qui n’ont pas été approuvés expressément par Futurecom Systems Group ULC pourraient faire perdre à l’utilisateur son droit à utiliser cet équipement.

# Informations de Sécurité Importantes

Le répéteur DVRS est conçu pour être utilisé dans des conditions professionnelles/contrôlées, dans lesquelles les utilisateurs connaissent à fond leur exposition et peuvent exercer le contrôle nécessaire sur celle-ci pour se conformer aux limites de la FCC/ISED. Cette radio N’EST PAS autorisée pour être utilisée par le grand public, les consommateurs ou autres.

# Avis Aux Utilisateurs (FCC/ISED)

Pour satisfaire les exigences de la FCC/d’ISED en matière d’exposition à l’énergie RF pour les transmetteurs mobiles, prière de consulter la Brochure Sécurité RF pour obtenir le facteur d’utilisation transmission/réception et la distance de séparation entre l’antenne de cet appareil et les personnes pendant l’utilisation. Pour assurer la conformité, le fonctionnement à une distance moins élevée n’est pas autorisé.

Futurecom demande à l’opérateur du répéteur P25 DVRS de satisfaire aux exigences de la FCC/ISED en matière d’exposition à l’énergie RF. La distance minimale entre toutes les personnes possibles et une antenne omnidirectionnelle doit respecter les indications de la Brochure Sécurité RF. Tout manquement à respecter la zone d’exclusion autour de l’antenne définie par la distance correspondant à la limite d’exposition maximale peut exposer les personnes qui se trouvent dans ce rayon à une énergie RF supérieure à la limite d’exposition de la FCC pour les spectateurs (population générale).

C’est à l’opérateur du répéteur qu’il incombe de s’assurer que les limites d’exposition maximales sont respectées en tout temps pendant les transmissions du répéteur. L’opérateur du répéteur doit s’assurer en tout temps que personne ne s’approche de l’antenne à une distance inférieure à celle correspondant à la limite d’exposition minimale.

# Proprietary Statement

2021 Futurecom Systems Group ULC.
All Rights Reserved

Futurecom, PDR8000®, DVR-LX®, the Futurecom Logo and the Stylized FC logo are registered trademarks of Futurecom Systems Group, ULC. All other trademarks are the property of their respective owner.

This document is provided as is. This document may contain technical inaccuracies or typographical errors. Futurecom is not responsible for any changes made from original format.

Futurecom Systems Group ULC. reserves the right to make changes or improvements to the equipment, software or specification described in this document at any time and without prior notice. These changes will be incorporated in the new releases of this document.

Futurecom Systems Group ULC. waives responsibility for any labour, materials or costs incurred by any party as a result of using this document.

MOTOROLA, MOTO, MOTOROLA SOLUTIONS and the Stylized M Logo are trademarks or registered trademarks of Motorola Trademark Holdings, LLC and are used under license. All other trademarks are the property of their respective owners.

# Getting Started

## How To Use This Guide

This User’s Guide covers the basic operation of the Futurecom line of Vehicular Repeaters. It applies to the following products: VRX1000, DVR and DVR-LX®.

VRX1000                                                           DVR                                                     DVR-LX®

## Notations Used In This Guide

Throughout the text in this guide, the following notations are used for emphasis:

DANGER: An operational situation which may result in injury or death. Immediate user action required.

WARNING: An operational procedure that must be accounted for to ensure equipment is working properly.

NOTE: Key supplemental information for user.

CONFIGURABLE: User experience may differ depending on how equipment has been configured.

# System Topology

A system typically involves the following components: Local Portable Subscriber Unit (PSU), Vehicular Repeater (VR), Mobile Subscriber Unit (MSU), System Tower, System PSU, and Dispatcher. Two key communication paths, VR Channel and MSU Channel, are referenced throughout the document. VR Channel is used to refer to the path between local PSUs and the VRs. MSU Channel is used to refer to the path between the MSU and the System. Both communication paths and all components are identified in the figure below.

SYSTEM TOPOLOGY

NOTE: For a Trunking System, the communication path between the MSU and the System is referred to as a Talkgroup. Throughout this document, the Conventional System term, Channel, will be used.

# VR Introduction

The VR is used to extend portable radio communications. When PSUs have difficulty reaching a system tower, a VR can help by linking the PSU to a higher power mobile radio or MSU.

LEGEND

PSU unable to reach radio tower site

PSU able to reach radio tower site through VR

VR     
interfaced
with MSU

POLICE VEHICLE MOUNT SCENARIO: PSU COVERAGE RANGE- WITHOUT vs WITH VR

FIRE VEHICLE MOUNT SCENARIO: PSU COVERAGE RANGE- WITHOUT vs WITH VR

LEGEND

PSU unable to reach radio tower site

PSU able to reach radio tower site through VR

VR     
interfaced
with MSU

RACKMOUNT SCENARIO: PSU COVERAGE RANGE- WITHOUT vs WITH VR

SUITCASE MOUNT SCENARIO: PSU COVERAGE RANGE- WITHOUT vs WITH VR

# VR Modes

The mode of the VR determines which communication paths are available to the user. There are three possible modes: OFF, LOCAL and SYSTEM.

Available modes depend on configuration.

OFF Mode (Mobile Mode)
USED WHEN: This mode is always used when a vehicle is moving- the VR is off.

COMMUNICATION PATHS: This mode allows the user to communicate with Dispatcher and System PSUs via MSU.

LOCAL Mode
USED WHEN: This mode may be used when a vehicle is stationary- the VR is active. This mode is typically used at a scene outside of network coverage and eliminates the system access deny tones heard on the local PSUs. This mode is also used when Dispatch does not need to monitor communications. It allows communication at a scene without using system resources- when MSU channels are limited.

COMMUNICATION PATHS: This mode allows communication between the local PSUs and MSU. With a DVR/DVR-LX®, there is repeated communication between local PSUs and MSU. With a VRX1000, the local PSUs communicate directly with each other and a local PSU communicates with the MSU through the VR.

Ability to monitor system activity on the selected MSU channel through the MSU speaker/local PSU is configurable.

SYSTEM Mode
USED WHEN: This mode may be used when a vehicle is stationary- the VR is active. This mode is used when communication or monitoring by Dispatch at a scene is required.

COMMUNICATION PATHS: This mode allows communication between the local PSUs, MSU, Dispatcher and System PSUs. With a DVR/DVR-LX®, there is repeated communication between local PSUs and MSU. With a VRX1000, the local PSUs communicate directly with each other and through the DVRS to communicate with Dispatcher and System PSUs.

Available modes depend on configuration. Local mode monitoring depends on configuration as well.

DVR/DVR-LX® MODES

Available modes depend on configuration. Local mode monitoring depends on configuration as well.

VRX1000 MODES

# DVRS Equipment

When a VR is interfaced with an MSU, the complete equipment package is referred to as a Digital Vehicular Repeater System (DVRS). The VR connects directly to the MSU and essentially becomes part of that MSU. The MSU continues to operate while the VR is active. All control of the VR is achieved through the MSU control head and all related messaging is displayed there as well.

VRX1000

CONTROL HEAD

MSU

DVR

CONTROL HEAD

MSU

DVR-LX®

CONTROL HEAD

MSU

DVRS PACKAGES- VR, MSU, CONTROL HEAD

## DVRS Configurations

DVRS equipment is available in a number of different mounting configurations: rack (DVR-LX® only), suitcase or vehicle.

Rackmount (DVR-LX®): This configuration is permanently installed in an indoor rack. The rack accomodates all components of the DVRS.

Suitcase Mount: This configuration packages all components of the DVRS in a durable suitcase. This unit can be deployed where needed and is powered up either by plugging into an AC outlet, or DC source such as vehicle or Battery Pack (sold seperately).

Vehicle Mount: This configuration typically involves installation of the control head in the front of the vehicle for user access while the VR and MSU are installed in the trunk, back seat or wherever space allows.

RACKMOUNT, SUITCASE MOUNT, AND VEHICLE MOUNT

# VR Operation

## How To Power Up VR

To power up the VR, turn on the MSU control head. The VR powers up together with the MSU.

The VR mode and VR channel that the VR powers up on is configurable.

NOTE: There is no independent power ON/OFF for a VR. The VR always powers up together with MSU.

## How To Activate VR

If the MSU is powered up and is on a VR Disabled MSU channel, the VR must be activated to communicate with the Dispatcher, System PSUs or other local PSUs. To activate, select a VR Enabled MSU channel.

Availability of VR Disabled channels is configurable.

If the VR is OFF, it must be activated to communicate with the Dispatcher, System PSUs or other local PSUs. It can be activated by one of the following methods:

ACTIVATING VIA MSU CONTROL HEAD SINGLE BUTTON PRESS  May be configured as a button press or long button press

STEP 1: Ensure MSU is powered up and a VR Enabled MSU channel is selected on control head.

STEP 2: Press the DVRS button.

ACTIVATING VIA MSU CONTROL HEAD MENU

STEP 1: Ensure MSU is powered up and a VR Enabled MSU channel is selected on control head.

STEP 2: Press DVRS button to access menu items.

STEP 3: Press MODE button to move through the available VR modes.

STEP 4: Press SEL button when desired mode is displayed or wait until the DVRS menu screen times out and the last selected mode is saved.

OR   NOTE: Navigation details depend on the control head

STEP 3: Use the up and down buttons on control head to move through the available VR modes.

STEP 4: Press SEL button when desired mode is highlighted or wait until the DVRS menu screen times out and the last selected mode is saved.

AUTOMATIC VEHICULAR REPEATER ACTIVATION (AVRA)    Only if configured; specific set of actions may vary.

If configured, the VR may be activated by a predetermined set of actions such as putting the vehicle into park and opening the driver’s side door. The user still has the option of manually activating the VR from the MSU control head or using the PSU remotely.

REMOTE ACTIVATION VIA LOCAL PSU

There are two options for remotely activating a VR via a local PSU.

- send a Call Alert Page using the ID of the specific VR

- initiate an Emergency Alarm

- enter a predefined Touch-Tone sequence (Analog PSUs only)

REMOTE ACTIVATION VIA SYSTEM USER

The Dispatcher or System PSU can send a Call Alert Page to a specific MSU ID which will cause its attached VR to cycle through modes in the following order: OFF-SYS-LOC-OFF. The Dispatcher can receive notification of the mode change for the VR as a Status message, if enabled.

List of available modes depends on configuration. Mode change notification for Dispatcher is also configurable.

How To Deactivate VR

If the VR is active (VR SYSTEM MODE or VR LOCAL MODE depending on available modes), it must be deactivated before driving to/away from an incident. It can be deactivated by one or more of the following methods:

DEACTIVATING VIA MSU CONTROL HEAD SINGLE BUTTON PRESS  May be configured as a button press or long button press.

Press the DVRS button.

DEACTIVATING VIA MSU CONTROL HEAD MENU

STEP 1: Press DVRS button to access menu items.

STEP 2: Press MODE button until VR OFF mode is displayed.

STEP 3: Press SEL button or wait until the DVRS menu screen times out and the last selected mode is saved.

OR NOTE: Navigation details depend on the control head

STEP 3: Press SEL button when desired mode is highlighted or wait until the DVRS menu screen times out and the last selected mode is saved.

DEACTIVATING VIA MSU CHANNEL SELECTION ON CONTROL HEAD  If VR Disabled channels are configured

Select a VR Disabled MSU Channel using the control head.

NOTE: If the MSU Channel is VR Disabled, a VR DISABLED status message is displayed on the top line of the control head and there is no VR status icon visible.

AUTOMATIC DEACTIVATION   Only if configured; specific set of actions may vary

AVRA- The VR may be deactivated by a predetermined action e.g.: placing the PSU back in the charger, shifting out of park or releasing air brakes. The user still has the option of manually deactivating VR from MSU control head or using the PSU remotely.
Inactivity Timer- If enabled, the VR will switch to OFF mode after a preconfigured amount of time.

GPS Driven Deactivation- The VR may be deactivated by exceeding a preconfigured threshold related to speed and/or distance.

REMOTE DEACTIVATION VIA SYSTEM USER

The Dispatcher or System PSU can send a Call Alert Page to a specific MSU ID which will cause its attached VR to cycle through modes in the following order: OFF-SYS-LOC-OFF. `The Dispatcher can receive notification of the mode change for the VR as a Status message, if enabled.

List of available modes depends on configuration.

## How To Power Down VR

To power down the VR, turn off the MSU control head. The VR powers down together with the MSU.

NOTE: There is no independent power ON/OFF for a VR. The VR always powers down together with MSU.

## How To Navigate Control Head Menu

The MSU control head provides access to the VR. The top line of the display contains VR information while the second line of the display contains MSU information. At the bottom of the display there are options that are accessed through the buttons below.

CONTROL HEAD DISPLAY

### VR MODE

This is the current mode of the VR: VR OFF (Off/Mobile Mode), VR SYS (System Mode) or VR LOC (Local Mode).

List of available modes depends on configuration.

### VR CHANNEL

This is the channel that the VR and the PSU use for communication. The local PSU must be on this channel to communicate through the DVRS.

NOTE: The VR can only be activated when a compatible VR Enabled MSU channel is selected. If a VR Disabled MSU channel is selected, the control head will display a “VR DISABLED” status message.

Availability of VR Disabled channels is configurable.

### VR STATUS ICON

The following status icons relate to the VR operation:

PRIMARY/SECONDARY INDICATORS

| DISPLAY ICON | VR STATUS | DESCRIPTION |
| --- | --- | --- |
| Steady | Primary (Active) | This VR is responsible for all communications in its coverage area. All other VRs in the coverage area are in Secondary state. |
| Flashing | Secondary (Standby) | This VR is not responsible for communications. There is a Primary VR already present in the coverage area. |
| Steady | Active Permanent Primary | This VR is responsible for all communications in its coverage area. All other VRs in the coverage area are in Secondary state. NOTE: If there was a Primary VR in the coverage area, it is now in Secondary state. Access to Permanent Primary is configurable. |
| Flashing | Standby Permanent Primary | This VR is not responsible for communications. There is an Active Permanent Primary already present in the coverage area. NOTE: When the current Active Permanent Primary leaves the coverage area, this VR will become the new Active Permanent Primary. Access to Permanent Primary is configurable. |

#### TRANSMIT/RECEIVE INDICATORS

| DISPLAY ICON | VR STATUS | DESCRIPTION |
| --- | --- | --- |
|  | VR Receiving | This VR is currently receiving voice/data. |
|  | VR Transmitting | This VR is currently sending voice/data. |
|  | VR Receiving and Transmitting | This VR is currently sending and receiving voice/data. |

NOTE: The Transmit/Receive arrow indicators are overlaid on top of the Primary/Secondary indicators on the control head.

### VR STATUS MESSAGES

The following status messages relate to the VR operation:

#### STATUS MESSAGES

| DISPLAY MESSAGE | VR STATUS DESCRIPTION | ACTION |
| --- | --- | --- |
| VR SERVICE | VR is in Service Mode and is not operational. | Wait until current activity has completed. The status message will clear once VR is operational. |
| VR LIMITED | VR has limited functionality while MSU is roaming. | Wait until MSU is back within range of its home system and the status message will clear. |
| VR ERROR NOTE: Exact error message may vary | VR detected error. Display toggles between VR ERROR and the VR Mode and Channel info. | Radio technician support required to resolve error. |
| VR IN RESET | VR is being restarted and is not operational. | Wait until restart is complete. The status message will clear once VR is operational. |
| VR DISABLED | VR is disabled on selected MSU Channel and is not operational. Only if VR Disabled channels are configured. | Select a VR Enabled MSU Channel and the status message will clear. |
| VR WARNING NOTE: Exact warning message may vary | VR detected warning. Display toggles between specific warning message (i.e. TRANSL TG WARN) and the VR Mode and Channel info. | Radio technician support required to resolve warning. |

STATUS MESSAGE DISPLAY EXAMPLE- VR SERVICE

DVRS BUTTON

The DVRS button provides the ability to change VR Mode and VR Channel. The Button Press is used to [insert configuration here] and the Long Button Press is used to [insert configuration here].

Description depends on how configured. For example: Button press to toggle VR between OFF/Active mode; Long button press to access DVRS menu.

NOTE: Active mode can be SYSTEM or LOCAL depending on how configured.

HOW TO CHANGE VR MODE THROUGH SINGLE BUTTON PRESS

HOW TO CHANGE VR MODE THROUGH DVRS MENU

STEP 1: Press the DVRS button.

STEP 2: To select a VR mode, use the up and down buttons on the control head to highlight the desired mode. When desired mode is highlighted, press the SEL button or wait until the menu screen times out and the last selected mode is saved.

OR

STEP 2: To select a VR mode, press the MODE button to scroll through the available modes. When the desired mode is displayed, press the SEL button or wait until the DVRS menu screen times out and the last displayed mode is saved.

NOTE: Navigation details depend on the control head

HOW TO CHANGE VR CHANNEL THROUGH DVRS MENU

STEP 1: Press the DVRS button.

OR

STEP 2: To select a VR channel, use the up and down buttons on the control head to highlight the desired channel. When desired channel is highlighted, press the SEL button or wait until the menu screen times out and the last selected channel is saved.

STEP 2: To select a VR channel, use the navigation buttons or rotary knob on the control head to scroll through the available channels. When desired channel is displayed, press the SEL button or wait until the menu screen times out and the last

displayed channel is saved.

NOTE: Navigation details depend on the control head

HOW TO ENTER/LEAVE PERMANENT PRIMARY STATE THROUGH DVRS MENU  Option available if configured.

OPTION 1, STEP 1: Press the DVRS button.

OPTION 1, STEP 2: Press the Permanent Primary (PM) button.

OPTION 1, STEP 3: Display will indicate that state has been changed successfully. Press EXIT button or wait until the menu screen times out to return to main menu.

OR

OPTION 2, STEP 1: Long Press the DVRS button.

OPTION 2, STEP 2: Display will indicate that state has been changed successfully. Press EXIT button or wait until the menu screen times out to return to main menu.

### VR STATUS TONES

The VR is not only an audio extender, but a network extender, which means the user will not only hear status tones on the MSU speaker but will also hear PSU tones like what is heard when talking directly on the trunked system. The following two tables indicate the VR specific tones that are heard under the described circumstances.

#### VR STATUS TONES HEARD FROM MSU SPEAKER

| TONE NAME | DESCRIPTION | TONE TYPE |
| --- | --- | --- |
| Invalid Menu Button Press | Indicates when a button is pressed for an invalid menu option. | Short low-pitched tone |
| Permanent Primary ON/OFF | Indicates when VR enters Permanent Primary state and when it leaves Permanent Primary state. Only if Permanent Primary is configured. | Chirping tone |
| Valid Menu Button Press | Indicates when a button is pressed for a valid menu option. | Short high-pitched tone |
| VR Control Timeout | Indicates the automatic exit of the DVRS Menu. This occurs after a period of inactivity. Period of inactivity is configurable; may also be completely disabled. NOTE: The last DVRS settings (VR channel, VR mode, Permanent Primary status) selected by the user and displayed on the MSU control head become active. | Long high-pitched tone |
| VR Mode Change | Indicates the VR has changed modes. - Via Control head - Via AVRA/GPS | - One quick tone followed by 2 long tones - One long tone |
| 50% Transmit Duty Cycle | Indicates that the DVRS has been transmitting more than 50% of the time over the past 30 minutes. DANGER: This may result in excessive RF radiation for any nearby bystanders. The users must immediately limit their talk time to comply with FCC rules. | Two short low-pitched tones |

#### COMMON VR STATUS TONES HEARD FROM VR ENABLED PSU

| TONE NAME | DESCRIPTION | TONE TYPE |
| --- | --- | --- |
| VR Activation Tones | Indicates that the VR has been activated. | Three slow, long tones |
| PSU Go-Ahead Tones: Local Access | Indicates when user can begin talking after PTT on PSU. NOTE: VR in Local Mode and will only be heard by local PSUs. Only if Local Mode available and tone has not been disabled. | Three very quick, very short chirps Lower pitch than System Access |
| PSU Go-Ahead Tones: System Access | Indicates when user can begin talking after PTT on PSU NOTE: VR in System Mode and will be heard by Dispatcher, System PSUs and local PSUs. Only if System Mode available. | Three very quick, very short chirps Higher pitch than Local Access |
| PSU Fallback (Talkaround) | Upon longer PTT on PSU, indicates PSU is out of range of the VR but direct communication with other local PSUs is possible. i.e. PSU is in Talkaround mode. | One very long tone followed by four very quick, very short chirps |
| PSU Out of Range | Upon PTT on PSU, indicates that PSU is out of range of the VR and communication to Dispatcher, System PSUs or other local PSUs is not possible. | One very long tone |

DVRS Suitcase Deployment

The DVRS Suitcase can be deployed in locations where other configurations – rack or vehicle mount – are not practical.

## Deployment Instructions

STEP 1: Connect the MSU and VR antennas to the corresponding connection ports.

STEP 2: Place the MSU antenna in a location for optimal system coverage.

STEP 3: Place the VR antenna in a location for optimal local PSU coverage.

STEP 4: Ensure the two antennas are separated to provide 30dB minimum isolation to prevent interference.

STEP 5: Connect the power supply either to AC or Battery Pack by using the corresponding cables.

STEP 6: Open the top cover and power up the MSU. Select the required MSU Channel, VR Channel and VR Mode.

STEP 7: Exchange test calls with a local PSU to ensure communication is successful.

Figure 1 - DVRS Suitcase Connections

WARNING: The DVRS Suitcase lid must be open while the unit is in operation to ensure adequate cooling.

NOTE:

Crossband Configuration – MSU Antenna Connection Port A and MSU Antenna Connection Port B are both used for connection to mobile.

In-band Configuration – MSU Antenna Connection Port A is used for Crossband connection, and MSU Antenna Connection Port B is used for In-band connection.

STEP 8: If necessary, change the location of the antennas for improved radio coverage.

MSU ANTENNA CONNECTION

PORT A

VR ANTENNA CONNECTION

MSU ANTENNA CONNECTION PORT B

BATTERY PACK CONNECTION

AC CONNECTION

It is not recommended to connect the battery pack when the Suitcase is powered from AC. A discharged battery pack can be damaged. Connecting the battery pack and AC at the same time voids battery pack warranty.

# DVRS Feature Overview

The DVRS offers an extensive set of features with numerous configuration options. This section contains a summary of key user features along with a brief description. For a complete list of features and complete list of configuration options, refer to supplementary Futurecom documentation.
 Feature availability is configurable.

## DVRS FEATURE DESCRIPTIONS

| FEATURE | DESCRIPTION |
| --- | --- |
| VR Enabled & Disabled MSU Channels | When a VR Enabled channel is selected on the MSU, messages are repeated between system users of the selected channel (Dispatcher, System PSUs) and PSUs on the selected VR channel. When a VR Disabled MSU channel is selected, all VR functions are disabled. |
| Emergency Revert | When VR receives an Emergency Alarm/Call from a PSU, it changes (steers) the currently selected MSU channel to a preconfigured channel. After cancellation of the Emergency from the PSU, the VR steers the MSU back to the original channel. Preconfigured channel name is configurable. |
| In Car Monitor | Allows user to monitor PSU voice traffic on the MSU speaker. User will hear PSU voice from selected VR enabled channel or all channels, depending on configuration. |
| MSU Fallback (Local Tx) | When MSU fails to access the system, the MSU microphone audio is re-routed and transmitted locally by the VR. |
| MSU Scan | The following will occur when MSU scan is enabled and a VR Enabled channel is selected: - PSU initiated call will always move MSU from a landed channel back to the selected channel. - Emergency Alarm/Call from a landed channel is passed through to the PSU. - If landed channel is VR Disabled, call is not passed through to the PSU. |
| Permanent Primary | Allows for manual override of the automatically selected Primary. A Permanent Primary VR will always become the Primary VR in a coverage area and will be responsible for all communications. Useful for large scenes where the location of the Primary VR becomes strategic to ensuring communication reaches all local PSUs. VR may be left with Permanent Primary designation set or, if configured, press the PM button on control head to toggle Permanent Primary status ON/OFF. |
| Primary/Secondary | This fully automatic feature ensures interference-free communication with no user input required. When multiple VRs are in the same coverage area, one unit will be designated as Primary and is responsible for all communications. All other VRs in the coverage area become Secondary units and enter a standby state. Designation is displayed within the Status Icon on the control head. WARNING: For vehicle-mount, VR must be OFF, or a VR Disabled MSU channel selected, when vehicle is moving. |
| PSU Remote VR Activation & Steering | Local PSU can send a Call Alert Page to a specific VR and remotely change the following: - VR Mode - to enable PSU communication through VR with Dispatcher and System PSUs - Classic Operation: mode change from OFF/LOCAL to System - Flexible Operation: cycle thru a set of preconfigured modes - VR Status (from Secondary to Primary) - to ensure PSU communicates through specific VR rather than a distant VR as distant VR access may result in degraded audio - MSU Selected Channel - to allow user to change selected MSU channel and communicate with a different group of users Available modes and channel steering are configurable. |

# References

www.futurecom.com → Support → Documentation and Software

- VR Functional Description:

DVR-LX/DVR/VRX1000 → Functional Description → VR Functional Description

- VR Programming Guide

DVR-LX/DVR/VRX1000 → Installation & Programming Guide → VR Programming Guide

- DVR Installation Guide:

DVR-LX/DVR/VRX1000 → Installation & Programming Guide → DVR Installation Guide

- Suitcase DVRS Installation Guide:

DVR-LX/DVR/VRX1000 → Installation & Programming Guide → Suitcase DVRS Installation Guide

- DVR-LX® Installation Guide

DVR-LX/DVR/VRX1000 → Installation & Programming Guide → DVR-LX® Installation Guide

- Futurecom How To Video Series:

Futurecom.com → click on YouTube icon at top of page

- RF Safety Booklet/ la Brochure Sécurité RF (Canada):

DVR-LX/DVR/VRX1000 → RF Safety Booklet → RF Safety Booklet – APX – DVRS - Canada

- RF Safety Booklet/ la Brochure Sécurité RF (USA/États-Unis):

DVR-LX/DVR/VRX1000 → RF Safety Booklet → RF Safety Booklet – APX – DVRS - US

Appendix A: All VR Status Tones- P25 PSU

The following DVRS specific tones are available in the P25 VR Enabled PSU:  Tones available only if related mode is configured.

| TONE NAME | DESCRIPTION | TONE TYPE |
| --- | --- | --- |
| Go Ahead Local (clear) | Indicates VR is in Local mode and local PSU can begin clear (unencrypted) voice call. | Series of short tones |
| Go Ahead Local (secure) | Indicates VR is in Local mode and local PSU can begin secure (encrypted) voice call. If encryption is configured. | Series of short tones |
| Go Ahead System (clear) | Indicates VR is in System mode and local PSU can begin clear (unencrypted) voice call. | Series of short tones |
| Go Ahead System (secure) | Indicates VR is in System mode and local PSU can begin secure (encrypted) voice call. If encryption is configured. | Series of short tones |
| Low Battery | Indicates battery voltage has dropped below the configured threshold. Threshold value is configurable | Low pitched tone every 30 seconds |
| Inactivity Timer | Indicates that VR will switch from SYSTEM/LOCAL to OFF. Sent once a minute for the last 5 minutes prior to switching off. | Two short tones |
| Queued Call | Indicates local PSU call has been queued, waiting for Go Ahead System tone. | Series of short tones |
| VR Active | Indicates the VR has been activated. | Three tones |

NOTE: A P25 Generic PSU will support the following VR status tones: VR Active, Low Battery and No Activity Timer Expiration.

Appendix B: All VR Status Tones- Conventional Analog PSU

The VR in analog mode can be configured to send the following tones to the analog local PSUs:

| TONE NAME | DESCRIPTION | TONE TYPE |
| --- | --- | --- |
| Emergency ACK | Indicates VR received an MDC1200 emergency issued by a local PSU. | Configurable or default (three short tones) Tone is configurable. |
| Encrypted Call | Indicates MSU is receiving an encrypted call. The tones are heard until the encrypted call ends. | One tone |
| Leading Deny Leading Go Ahead | Indicates if channel is available for local PSU voice call. Initial PTT on Local PSU is released to wait for the leading tone. If Go Ahead tone is heard, then local PSU may proceed with second PTT and begin voice call. | Long low-pitched tone Three short tones |
| Low Battery | Indicates battery voltage has dropped below the configured threshold. Threshold value is configurable. | Single low-pitched tone every 30 seconds |
| No Activity Timer Expiration | Indicates that VR will switch from SYSTEM/LOCAL to OFF. Sent once a minute for the last 5 minutes prior to switching off. Available modes are configurable. | Two short tones |
| Trailing Access Denied | Indicates system was not reachable and PSU call was only repeated locally. No further PTT attempts will be successful. Radio technician support required to resolve access issue. Tone heard after local PSU PTT released. NOTE: Lack of a trailing tone indicates that PSU user is outside of VR range. NOTE: Local repeat of PSU call only applicable to DVR/DVR-LX®; not for VRX1000 | Single low-pitched tone |
| Trailing Successful System Access | Indicates transmission was successful and PSU call was sent to System Users. Tone heard after local PSU PTT released. NOTE: Lack of trailing tone indicates that PSU user is outside of VR range. | Single high-pitched tone |
| Trailing Unsuccessful System Access | Indicates system was not reachable and PSU call was only repeated locally. Subsequent PTT attempts may be successful. Tone heard after local PSU PTT released. NOTE: Lack of trailing tone indicates that PSU user is outside of VR range. NOTE: Local repeat of PSU call only applicable to DVR/DVR-LX®; not for VRX1000. | Longer low-pitched tone |
| VR Activation | Indicates the VR has been activated. | Configurable number of tones, tone duration and frequencies Tone is configurable. |

NOTE: All PSUs which are in the analog mode VR radio coverage area and on the VR channel will hear VR status tones.

# Glossary

| TERM | DEFINITION |
| --- | --- |
| Active Permanent Primary | This VR is responsible for all communications in its coverage area. All other VRs in the coverage area are in Secondary state. A Permanent Primary VR has higher priority than a Primary VR- forces a Primary VR into Secondary state. |
| AVRA | Automated Vehicular Repeater Activation. An option which enables the activation and deactivation of the repeater through a predefined trigger- such as automatically activating the repeater when the vehicle is put into in park and the driver door opens. This requires additional equipment/ installation effort. |
| Channel | The communication path between VR and PSU, or MSU and Conventional System. |
| DVR | Digital Vehicular Repeater |
| DVRS | Digital Vehicular Repeater System- VR interfaced to a mobile radio with a control head. |
| GPS | Global Positioning System |
| Local Mode | DVR/DVR-LX®: mode which allows repeated communication between the local PSUs and MSU. If configured, can also monitor activity on the selected MSU channel. VRX1000: mode which allows non-repeated communication between local PSUs and MSU. |
| Local PSU | PSU switched to the VR channel and used for communication through the VR. |
| MSU | Mobile Subscriber Unit (aka mobile radio) |
| MSU Fallback (Local Tx) | When MSU fails to access the system, microphone audio is re-routed and transmitted locally by the VR. |
| Primary VR | This VR is responsible for all communications in its coverage area. All other VRs in the coverage area are in Secondary state. |
| PSU | Portable Subscriber Unit (aka portable radio) |
| PTT | Push To Talk |
| Secondary VR | This VR is not responsible for communications. There is a Primary VR already present in the coverage area. |
| Standby Permanent Primary | This VR is not responsible for communications. There is an Active Permanent Primary already present in the coverage area. If the Active Permanent Primary leaves, the Standby will become the new Active Permanent Primary VR. |
| System Mode | DVR/DVR-LX®: mode which enables repeated communication between the local PSUs, MSU and two-way communication with Dispatcher and System PSUs. VRX1000: mode which allows non-repeated communication between local PSUs and communication with Dispatcher and System PSU. |
| System PSU | A portable subscriber unit that communicates directly with the system. |
| Talkgroup | A group of radio users who communicate with each other by using the same communication path on a Trunking System. |
| Touch-Tone | Term used to refer to Dual Tone Multi Frequency (DTMF) tones generated by pushing button(s) on PSU keypad. |
| VR | Vehicular Repeater- generic term used to refer all different models of repeaters that this guide applies to. |
| VR Disabled MSU Channel | A channel selected on the MSU that has not been configured to work with a VR. |
| VR Enabled Channel | A channel selected on the MSU that has been configured to work with a VR. |
| VR Mode | Determines the communication capabilities between System Users and Local Portable Users; Can be set to OFF, LOCAL or SYSTEM. |

Subscribe to our newsletter if you want to be informed about new releases and updates.

Please visit http://futurecom.com/support/newsletter/

Futurecom Systems Group, ULC.

3277 Langstaff Rd

Concord, Ontario L4K 5P8

Canada

1-800-701-9180

https://www.futurecom.com/

- © 2021 Futurecom Systems Group, ULC

[1] Refer to RF Safety Booklet (Canada) or RF Safety Booklet (USA) for separation distances.

[2] Refer to RF Safety Booklet (Canada) or RF Safety Booklet (USA) for separation distances.

Prière de consulter la Brochure Sécurité RF (Canada) ou la brochure Sécurité RF (États-Unis) pour les distances de séparation

Prière de consulter la Brochure Sécurité RF (Canada) ou la brochure Sécurité RF (États-Unis) pour les distances de séparation

Refer to Suitcase DVRS Installation Guide for deployment instructions.

Refer to VR Programming Guide for details on how to install AVRA option.

Refer to VR Programming Guide for details on how to install AVRA option.

Refer to VR Programming Guide for full details on this configurable option.

Refer to VR Programming Guide for full details on this configurable option.

Refer to DVRS Feature Overview section for further details on the Primary/Secondary and Permanent Primary features.

Refer to DVRS Functional Description for a complete list of unsupported PSU features.

Refer to DVRS Installation and Programming Guide for full details on error codes.

Refer to DVRS Installation and Programming Guide for full details on warning messages.

Refer to How To Video Series Episode 7 for demonstration of common VR status tones heard from PSU.

Refer to APPENDIX A: VR STATUS TONES- P25 PSU and APPENDIX B: VR STATUS TONES- CONVENTIONAL ANALOG PSU for a full list of status tones.

Refer to How To Video Series Episode 7 for demonstration of common VR status tones heard from PSU.

Refer to How To Activate VR section for description of activation methods.

Refer to DVRS Functional Description for a complete list of VR features.

Refer to DVRS Installation and Programming Guide for a complete list of configuration options.

Refer to How To Video Series Episode 5 for more details on Primary/Secondary feature.

Refer to How To Video Series Episode 1 for demonstration on how to remotely activate a VR (Classic Mode only).